Icon Updating

- Hold pressing PTT key and PF2 key (the bottom side key), at the same time power on radio, LCD display UPDATE MODE

- Use PC cable connect PC to radio

- In CPS-> Set ->Set Com, choose the correct Com Port.

- In CPS ->Tool, choose the firmware upgrade.

- Click Open Update File，choose .spi (either Anytone Start up or Welcome Start up).

- Com Speed  921600, tick on Duplex

- Click Write to update the radio the latest firmware.

- Reboot the radio when the update is finished.